[image: image1.jpg]SCG SMART CABLING & TRANSMISSION CORP.

10F, No0.493, Chung-Cheng Rd., Hsin Tien City, Taipei 231, Taiwan. R.0.C.
SMART CABLING & TRANSMISSION  Tel: 886-2-2218-6886 Fax 886-2-2218-7989 http:/ww.sct.com.tw E-maitservice@sct.com.tw





SDI05 Testing Report
Date: 7th  Feb 2013
RD Engineer: Kevin Cho
Testing model:  SDI05  HD-SDI over CAT5 (HD-SDI Passive Balun)
Test Equipment:
1. ASTRO VG-859C pattern generator, SDI01(SC&T HD-SDI to HDMI converter)+SDI02 (HDMI to HD-SDI converter) 
2. Taiwan Mintron Brand”1080p Camera+ SDI05 SDI over UTP cable, receiver end add “SC&T” SDI01 “HD-SDI to HDMI converter” for display
SD-SDI (270Mb/s) Testing：
Cable：CAT.5e UTP
Distance：130 meters
Resolution：1440x480i @59.94/60Hz、1440x576i @50Hz

HD-SDI (1.48Gb/s) Testing：
Cable：CAT.5e UTP
Distance：30 meters
Resolution：1280x720P @50/59.94/60Hz、1920x1080i @50/59.94/60Hz、1920x1080p @25/30Hz
Cable：CAT.6 UTP
Distance：45 meters
Resolution: 1280x720P @50/59.94/60Hz、1920x1080i @50/59.94/60Hz、1920x1080p @25/30Hz
Testing result: Using “Mintron Brand”1080p Camera+ SDI05 SDI over UTP cable, receiver end add “SC&T” SDI01 “HD-SDI to HDMI converter” for display，CAT.5e UTP 30 meters、CAT.6 UTP 45meters OK。
3G-SDI (2.97Gb/s) Testing：
No picture
[image: image1.jpg]