THE  SPECIFICATION  

OF

CAT.5E UTP PATCH 4 PAIRS CABLE

 1.Scope

 This specification describes a high speed data communication cable construction consisting of

Two twisted pair, and PE jacket. They are suitable for use in high speed up to 100MHz

ethernet  network system.

2.Safety Standards

   UL  TYPE  Style  Page  CM  complying  with  UL Subject 758,UL 444 and UL 1581

3.Constrution

   3.1 Configuration                               

                                                  

                                    PE JACKET

                                                   

                                    TWISTED PAIR

        Twisted Pairs Color Code

	 1
	Orange,White/Orange

	 2
	Blue, White/Blue

	 3
	Green,White/Green

	 4
	Brown,White/Brown


   3.1.1 Under 10,000hrs-UV.   

   3.2Component

       a.Size of conductor                   :AWG 24 (1 x 0.5mm) copper wire

       b.Dielectric material                  :PP

       c.Diameter of Dielectric core           :0.92 ± 0.03 mm

       d.UL style no. of single                :444  75 °C

       e.Number of twist pairs                :Two pairs.

                                            Each one of two pairs with different pitch

  3.3Assembly                              :Two pairs of dielectric core shall be stranded

                                             in circular form.  

  3.4Jacket  material                        :PE.

      Overall Diameter                      :ave.5.4± 0.2 mm

      Color                                : BLACK                                                           
