[image: image1.jpg]SCG SMART CABLING & TRANSMISSION CORP.

10F, No0.493, Chung-Cheng Rd., Hsin Tien City, Taipei 231, Taiwan. R.0.C.
SMART CABLING & TRANSMISSION  Tel: 886-2-2218-6886 Fax 886-2-2218-7989 http:/ww.sct.com.tw E-maitservice@sct.com.tw





 SPECIFICATION

	
	
	

	ITEM NO. 
	WT104PE
	[image: image1.jpg][image: image2.bmp]

	TYPE
	Outdoor CAT5E UTP4 pairs Twisted Pair CABLE
	

	Specification
	CAT 5E UTP+1.2Y
	

	Construction
	

	Conductor
	AWG
	CAT 5 UTP
	

	
	structure
	1/0.485(4P
	

	
	O.D. 
	0.485mm
	

	
	material
	BC
	

	Insulation
	Thickness
	0.2 mm 
	Self-support

	
	O.D.
	0.915 ( 0.05 mm
	Construction
	1.2mm wire

	
	Material
	PE
	
	

	
	Color
	(Blue、White/Blue)、(Orange、White/Orange) 
(Green、White/Green)、(Brown、White/Brown)

	
	CAT 5 UTP jacket: PVC Gray , O .D. 5.0 ( 0.2 mm

	Jacket
	Thickness
	0.80mm(0.7mm(Nominal)

	
	O.D.
	6.85(9.65(6.85(0.2 mm , 2.8(0.1 mm)

	
	Material
	PE

	
	Color
	Black


	Characteristic

	Item
	Characteristics

	Resistance（20℃）
	9.38 Ω Per 100m （328ft）

	D-C Resistance Unbalance （percent）,max
	5%

	Resistance Tests（20℃）
	1500MΩ-Km DC 500V

	Dielectric Tests
	DC 500V/1min.or AC 350V/1min.

	Capacitance
	Max.5.6Nf Per 100m（328ft）

	Capacitance unbalance
	330 Pf  Per 100m（328ft）

	Impedance(Ω)
	100±15Ω

	Frequency

（MHz）
	Attenuation

dB/100M）
	NEXT

（d/B100m）
	Structural Return Loss

（d/B100m）

	1
	2.0
	62
	23

	10
	6.5
	47
	23

	16
	8.2
	44
	23

	20
	9.3
	42
	23

	25
	10.4
	41
	22

	31.25
	11.7
	39
	21

	62.5
	17.0
	35
	18

	100
	22.0
	32
	16


Self-support





Jacket





CAT 5E UTP








