INDEX

System Requirement	1

iTunes Download Link	1

System Setting	1

Home Screen	2

Scan Devices	2

Add Clients	2

Modify Host	4

Select Client Channel	6

Modify Client	7

Delete Client	8

Add Group	9

Select Group Channel	10

Modify Group	12

Add Client of Group	14

Delete Group	15

Add Scene	16

Apply Scene	17

Modify Scene	19

Delete Scene	20

System Requirement

APP name: Remote Control Center PRO

iOS version 7.0 or later

Size: 5.1M

iTunes Download Link

https://itunes.apple.com/us/app/remove-control-center-pro/id930779117?l=zh&ls=1&mt=8

iTunes Download QR code

System Setting

Before setting, you have to connect Host / Client / Ethernet Switch / Access Point (Wi-Fi router) together.

Mobile phone connect to the Wi-Fi and set the IP address same with the Host / Client.

System default IP setting is Auto IP, it will assign 169.254.X.X (subnet mask 255.255.0.0) to transmitters and receivers, you could also set to DHCP or Static IP, please refer to user manual web setting chapter: IP Setup.

There are three ways to get the IP address of receiver:

1. Use APP function: Scan Device to get the IP address.

2. Local IP shows on right bottom screen when booting.

3. Press remote control button[MENU] [1] [ENTER](IP Address)

There are two ways to get the IP address of transmitter:

1. Use APP function: Scan Device to get the IP address.

2. Connect a transmitter and receiver and set in the same Channel, press remote control button [MENU] [1] [ENTER] at receiver side (Host IP Address), it will shows the transmitter IP Address on screen(must remove the HDMI cable of transmitter or turn off the video source).

Home Screen

When first time enter the APP will see the Client page

Top left「+」to add client

Top right「Edit」to delete client

Bottom left icon to enter Group/Scene mode selection page

Bottom right「Info」to enter Information page

Scan Devices

Touch top left「+」to enter Scan Devices page

Touch bottom left「Scan Device」to scan connected clients

Add Clients

After scan it will show all Clients in the list

Host will be add to bottom right Host page

Touch listed IP address to enter Add Device page to add Client

Enter the device Name you prefer and IP address

Notice: Name and IP should be unique, not allow duplicate

Touch「Add Device」button to finish setting.

Return to home screen will see the device which just add

Modify Host

Touch bottom right「Host」to enter Host setting page

Touch the gear icon of host to modify settings

Touch Name column to change name

Touch icon to select different icon

Touch top right「Save」to finish setting

Return to Host page will see the device which just changed

Select Client Channel

Left icon: Icon for host

Center: Upper big font for client, lower small font for host

Notice: if shows「Specify the host does not exist」means the host not exist

Right Ch.: Number of host channel, e.g.: Channel2 will show「Ch.2」

Right gear: Modify the client name

Touch anywhere except the gear to enter Select Host page

In Select Host page it will list all register host for selection

Touch the Host bar to select

The selected channel will be high light and switch the screen immediately

Modify Client

Touch the gear icon of client to modify settings

Enter the name in pop-up dialog, touch「Save」to finish

Return to home screen will see the device which just changed

Delete Client

Touch the top right「Edit」at Client page to enable select function

Check the client then touch the bottom right trash can icon to delete

Touch 「Delete」of pop-up dialog to confirm delete

Add Group

Touch the top right「Edit」at Client page to enable select function

Check the client then touch the bottom center「Save」

Enter the group name you prefer then touch「Group」to save

Notice: Group name could not be empty or duplicate

Touch top right「Done」to disable select function

Select Group Channel

Touch bottom left icon to enter Group/Scene mode selection page

Tough pop-up dialog「Group」to enter group channel selection

Touch group name enter Select Host page

Yellow area shows the devices of group

Green area shows the available channels

Touch the Host bar to select

The selected channel will be high light and switch the screen immediately

Modify Group

In Group page touch the gear icon to enter modify mode

Touch Name column to change group name

Drag the client name to left to show「Delete」button

Touch「Delete」to delete the client

Add Client of Group

Touch bottom left「+」in Modify Group page to add client

Check the client then touch top right 「Add」, it will return to Modify Group page

Return to Modify Group page will see the device which just add

Touch「Save」to save the modification

Delete Group

Touch top right「Edit」in Group page to show「-」button

Touch「-」button it will show 「Delete」button

Touch「Delete」button to delete the group

Touch top right 「Done」to exit

Add Scene

Touch the top right「Edit」at Client page to enable select function

Check the client then touch the bottom center 「Save」

Notice: if shows「Specify the host does not exist」means the host not exist and not able to add the client to Scene

Enter the scene name you prefer then touch「Scene」to save

Notice: Scene name could not be empty or duplicate

Touch top right「Done」to disable select function

Apply Scene

Touch bottom left icon to enter Group/Scene mode selection page

Touch 「Scene」to enter scene mode

Touch the scene name to apply

It will pop-up dialog「Scene have been applied」

Modify Scene

Touch the gear icon of scene in Scene page to modify

Enter new scene name in pop-up dialog then touch「Save」

Return to Scene page will see the name which just changed

Delete Scene

Touch top right「Edit」at Scene page to show「-」button

Touch 「-」to show「Delete」button

Touch 「Delete」button to delete the scene

Touch top right「Done」to exit