INDEX

System Requirement	1

Google Play Download Link	1

System Setting	1

Home Screen	2

Add Client	2

Modify Client	3

Modify Host	4

Select Channel of Client	5

Delete Client	6

System Requirement

APP name: Remote Control Center

Android 2.2 and up

Size: 1.7M

Google Play Download Link

https://play.google.com/store/apps/details?id=sct.com.remotecontrolcenter

Google Play Download QR code

System Setting

Before setting, you have to connect Host / Client / Ethernet Switch / Access Point (Wi-Fi router) together.

Mobile phone connect to the Wi-Fi and set the IP address same with the Host / Client.

System default IP setting is Auto IP, it will assign 169.254.X.X (subnet mask 255.255.0.0) to transmitters and receivers, you could also set to DHCP or Static IP, please refer to user manual web setting chapter: IP Setup.

There are three ways to get the IP address of receiver:

1. Local IP shows on right bottom screen when booting.

2. Press remote control button[MENU] [1] [ENTER](IP Address)

Home Screen

When first time enter the APP will see above home (Client) screen

Bottom left「+」to add client

Bottom center「Host」to enter host setting page

Bottom right「Info」to enter Information page

Add Client

Touch bottom left「+」to enter Add Device page

Enter the device Name you prefer and IP address

Notice: Name and IP should be unique, not allow duplicate

Touch「Add Device」button to finish setting.

Return to home screen will see the device which just add

Modify Client

Press client name at to show pop-up dialog, touch 「Modification」

Touch Name column to change name

Touch icon to select different icon

Touch top right「Save」to finish setting

Notice: IP address is not able to modify, please delete the client and add a new one if input wrong IP address

Return to home screen will see the device which just changed

Modify Host

Touch bottom center「Host」to enter host setting page

Press Host name to enter Modification page

Touch Name column to change name

Touch icon to select different icon

Touch top right「Save」to finish setting

Return to Host page will see the device which just changed

Select Channel of Client

Touch client name to enter channel selection page

Notice: it will show all 16 channels even does not exist

The selected channel will be high light and switch the screen immediately

Notice: it will switch to the channel even does not exist( just like a remote control)

Delete Client

Press Client name to show pop-up dialog

Touch「Delete」to pop-up dialog

Touch「Delete」to confirm