INDEX

System Requirement	1

iTunes Download Link	1

System Setting	1

Home Screen	2

Add Client	2

Modify Client	3

Modify Host	4

Select Channel of Client	5

Delete Client	6

System Requirement

APP name: Remote Control Center

iOS version 7.0 or later

Size: 2.2M

iTunes Download Link

https://itunes.apple.com/us/app/remote-control-center/id929873260?l=zh&ls=1&mt=8

iTunes Download QR code

System Setting

Before setting, you have to connect Host / Client / Ethernet Switch / Access Point (Wi-Fi router) together.

Mobile phone connect to the Wi-Fi and set the IP address same with the Host / Client.

System default IP setting is Auto IP, it will assign 169.254.X.X (subnet mask 255.255.0.0) to transmitters and receivers, you could also set to DHCP or Static IP, please refer to user manual web setting chapter: IP Setup.

There are three ways to get the IP address of receiver:

1. Local IP shows on right bottom screen when booting.

2. Press remote control button[MENU] [1] [ENTER](IP Address)

Home Screen

When first time enter the APP will see above home (Client) screen

Top left「+」to add client

Top right「Edit」to delete client

Bottom left「Host」to enter host setting page

Bottom right「Info」to enter Information page

Add Client

Enter the device Name you prefer and IP address

Notice: Name and IP should be unique, not allow duplicate

Touch「Add Device」button to finish setting.

Return to home screen will see the device which just add

Modify Client

Touch the gear icon of client to modify settings

Touch Name column to change name

Touch icon to select different icon

Touch top right「Save」to finish setting

Notice: IP address is not able to modify, please delete the client and add a new one if input wrong IP address

Return to home screen will see the device which just changed

Modify Host

Touch bottom left「Host」to enter host setting page

Touch the gear icon of host to modify settings

Touch Name column to change name

Touch icon to select different icon

Touch top right「Save」to finish setting

Return to host page will see the device which just changed

Select Channel of Client

Touch client name to enter channel selection page

Notice: it will show all 16 channels even does not exist

The selected channel will be high light and switch the screen immediately

Notice: it will switch to the channel even does not exist( just like a remote control)

Delete Client

Touch top right 「Edit」to show 「-」icon

Touch「-」icon to show「Delete」icon

Touch「Delete」icon to delete

Touch top right「Done」to exit